8/12/2026

I have found the most suitable and relevant paper for the work.

“Generic Attention-model Explainability for Interpreting Bi-Modal and Encoder-Decoder Transformers”

Why this fit with the work?

چون دقیقاً برای ترنسفورمرهایی طراحی شده که چند مُدال را در هم می‌آمیزند و relevance را به‌ازای هر توکن می‌دهد — یعنی همان سؤال «کدام توکن خروجی را جلو برد». اما صادقانه بگویم: این یک کتابخانه‌ی drop-in نیست؛ مخزن رسمی برای CLIP/LXMERT/DETR پیاده‌سازی شده و قاعده‌ی انتشار relevance را باید با ماژول attention خودتان تطبیق دهید. خبر خوب اینکه بخش زمانیِ شما فقط ۲ لایه و ۷ توکن است، پس هزینه‌ی تطبیق کم است.

Today, I analyzed the dataset.

This dataset captures the telemetry of an autonomous wheeled mobile robot equipped with an RGB-D camera, logged over a 49.9-second run at a stable ~10 Hz control-loop rate (497 samples, mean loop period ≈ 100 ms with only a single timing outlier at ~70 ms). Over the run the robot followed a curved, rotation-heavy path with a total travel distance of 12.36 m against a straight-line start-to-end displacement of only 2.1 m, turning through roughly 151° of heading while maintaining an average speed of about 0.25 m/s. Wheel commands were passed through the arbitration/safety layer largely unaltered (max command-vs-arbitrated deviation ≈ 80 RPM), the robot stayed in AUTO/permitted mode throughout, and target tracking remained tight, with a mean body-frame target distance of just 25 mm. The run terminated in a single controlled emergency stop on the final sample, where the state switched to ESTOP, motor commands dropped to zero, and permission was revoked.

Sensor synchronization was generally within one control cycle: RGB and depth streams averaged ~17 ms of sync error (occasionally spiking to 53 ms for depth and 67 ms for pose), while the RS485 link stayed very stable around 21 ms. The accompanying frames belong to sample 0 (timestamps match the rgb_time_ns/depth_time_ns fields) and show an outdoor dirt terrain with tire tracks and a treeline; the 16-bit depth map has ~15.4% invalid (zero-valued) pixels, concentrated in the sky and far field, which is expected for depth sensing in open outdoor scenes. Overall, the log indicates a well-behaved, well-synchronized autonomous run with accurate target following and a clean safety shutdown at the end.