ViT-Motion

Temporal Vision-Transformer Motion Model

Model & Dataset — Technical Report

Tracked-vehicle motion prediction from RGB + control history

Prepared for: APL

Project folder: Tasks/04  ·  Dataset day: 2026-07-30 (afternoon, wet)

Codebase version: v2 (temporal)

# Contents

In Word, right-click the list below and choose “Update Field” to fill in page numbers (Word may prompt automatically on open).

# 1. Executive summary

This project trains a neural network that watches a tracked vehicle (a tank-tread robot) drive over a wet surface and predicts how it will move in the next tenth of a second. At each step the model is given one camera image plus a short history of the vehicle's recent motion and wheel commands, and it outputs three numbers: how far the vehicle moves forward, how far it slides sideways, and how much it rotates — all expressed in the vehicle's own reference frame.

The codebase is the v2 “temporal” revision. The key idea of v2 is that motion is not memoryless: what the vehicle does next depends on what it was just doing. So instead of feeding the network a single snapshot, v2 feeds it a window of K = 5 consecutive time steps of numeric state, fused with a single visual token from the most recent image, through a small Transformer that mixes the tokens over time before predicting.

Current status: the data pipeline is built and verified — 18 driving experiments, ~8,100 clean samples, normalization and train/val/test splits are all generated. The model code, training loop, and evaluation/visualization tools are complete and pass a smoke test. What has not yet happened is a full training run: there is no trained checkpoint (best.pt) in the folder yet, so no accuracy numbers exist. The immediate next step is to run train.py.

At a glance  Input = 1 RGB image (224×224) + last 5 steps of [prev Δx, prev Δy, prev Δyaw, left speed, right speed].  Output = next-step [Δx, Δy, Δyaw] in the vehicle body frame.  Backbone = ViT-Tiny (pretrained) + 2-layer Temporal Transformer.  Loss = weighted Smooth-L1 on normalized targets.

# 2. The problem being solved

A tracked vehicle is commanded by two track speeds (left and right, in RPM). On a slippery/wet floor the vehicle does not move exactly the way a clean kinematic formula would predict — the treads slip, and the true displacement drifts from the commanded one. The goal is a learned motion model that maps “what the camera sees” and “what the tracks were commanded” to “what the vehicle actually did,” so it can later be used inside a controller (e.g. Model-Predictive Control) to plan accurate moves.

### 2.1  What the model predicts

Formally, one prediction is:

( RGB_t , n_{t-K+1 : t} )  →  ( Δx_t , Δy_t , Δφ_t )

      
where each numeric token is:
n_i = ( Δx_{i-1}, Δy_{i-1}, Δφ_{i-1}, v_L,i , v_R,i )

Δx = forward displacement (m), Δy = leftward displacement (m), Δφ (Δyaw) = change in heading (radians), over one time step — all in the current vehicle/body frame. These are small numbers: in a typical run Δx spans roughly −0.08 to +0.16 m, Δy ±0.05 m, and Δyaw ±0.09 rad per step.

Why body-frame deltas (not world coordinates)  The target is deliberately the change relative to the vehicle, not an absolute map position. This makes every sample location-independent (the model learns “given this state, how do I move,” not “where am I on the map”), and lets predictions be chained: accumulate the deltas to reconstruct a path.

# 3. The data

## 3.1  Where the data comes from

Each experiment starts life as a ROS2 recording (rosbag) from the robot: camera images, depth images, commanded track RPM, actuator feedback, and the SLAM pose estimate. A preprocessing step (upstream of this codebase) synchronizes those streams onto a fixed ~10 Hz clock (0.1 s period) and writes one folder per experiment. The summary.json in each folder records the source topics and sync statistics.

Per-experiment folder layout:

20260730_afternoon_wet_101/
  samples.csv        ← one row per synchronized time step (the ground truth table)
  summary.json       ← provenance: source bag, topics, sync-error stats, drop counts
  rgb/   <ns>.png     ← color frames, filename = camera timestamp in nanoseconds
  depth/ <ns>.png     ← aligned depth frames (NOT used by the model in v1/v2)

Experiment IDs follow YYYYMMDD_<time>_<surface>_<NNN>. This dataset is a single session: 2026-07-30, afternoon, wet, runs numbered 001–010 and 101–109. The real image files live on a shared network drive (\\192.168.111.168\APL_Share\…\ViT_data_tracked Vehicle); the CSV stores relative image paths and the pipeline resolves them to absolute paths.

## 3.2  The samples.csv schema

Each samples.csv has 45 columns and one row per time step (e.g. 497 rows for run 101). Only a handful of these columns feed the model; the rest are provenance, timing, and alternative signals kept for traceability. Grouped by purpose:

### Timing & indexing

| Column | Type | Meaning |
| --- | --- | --- |
| sample_index | int | Row counter within the experiment. Must be contiguous for a window to be valid. |
| command_step | int | Index of the command that produced this step. |
| time_sec | float | Seconds from the start of this experiment. |
| command_time_ns / target_time_ns | int (ns) | Absolute nanosecond timestamps of the command and of the resulting pose. |
| dt_sec | float | Time between this step and the next. Should be ≈ 0.1 s. Used as a quality filter. |

### Images & synchronization quality

| Column | Type | Meaning |
| --- | --- | --- |
| rgb_path / depth_path | str | Relative paths to the color and depth frames for this step. |
| rgb_time_ns / depth_time_ns | int | Timestamps of the chosen image frames. |
| rgb_sync_error_ms | float | How far the RGB frame's time is from the step time. Filtered at 40 ms. |
| depth_sync_error_ms | float | Same for depth (informational; depth is unused). |
| pose0/1_sync_error_ms | float | Timing gap of the two pose samples used to form the motion delta. |

### Control signals — three parallel sources

This is the part most worth understanding. The CSV records three versions of the track command, and choosing the wrong one silently corrupts training:

| Source (columns) | What it is |
| --- | --- |
| command_left/right_rpm | The raw commanded track speeds. This is what the model uses (config control_source: command). Range in run 101 ≈ ±3,500 RPM. |
| arbitrated_left/right_rpm (+ _source, _permitted, _emergency_stop) | The command after a safety/arbitration layer. arbitrated_source is AUTO in normal driving and ESTOP on an emergency-stop step. Selectable via config; when used, non-permitted and e-stop rows are dropped. |
| rs485_left/right_rpm (+ _source, _permitted, _emergency_stop) | Actuator-bus feedback (what the motor controller reported). In this data it is negated AND lagged by one step relative to the command — see the warning below. Never used as an input as-is. |

⚠  Data quirk you must not ignore  In these experiments rs485_left_rpm[i] ≈ −command_left_rpm[i−1] — the RS485 feedback has the opposite sign and is offset by one time step. If it were swapped in for the command columns, the model would train on wrong-signed, time-shifted inputs. The code guards against this by defaulting to the command source; the “arbitrated” path additionally filters emergency-stop / non-permitted rows. Bottom line: keep control_source: command unless you deliberately switch to arbitrated.

### Pose & the prediction target

| Column | Meaning |
| --- | --- |
| pose_x, pose_y, pose_z, pose_yaw | Current vehicle pose in the map/world frame (from ORB-SLAM). |
| next_pose_x…next_pose_yaw | Pose at the next step. The difference defines the motion. |
| target_world_dx / dy | Raw map-frame displacement (next minus current). |
| target_body_dx | TARGET #1: forward displacement in the current body frame (m). |
| target_body_dy | TARGET #2: leftward displacement in the current body frame (m). |
| target_delta_yaw | TARGET #3: wrapped heading change next−current (radians). |
| has_previous_delta | True once a valid previous transition exists. First row of each run is False. |
| previous_body_dx / dy / delta_yaw | The previous step's body-frame delta — fed back in as model input (zero on the first row). |

So the three target_body_* columns are the ground-truth answers, and the three previous_body_* columns are the same quantities from one step earlier, recycled as inputs so the model has short-term motion memory.

## 3.3  The 5 model inputs and 3 outputs

Out of 45 columns, the model actually consumes one image and five numbers per step, and predicts three:

| Role | Fields | Notes |
| --- | --- | --- |
| Image | latest rgb frame | Only the newest frame in the window is decoded (224×224, ImageNet-normalized). |
| Numeric ×5 | previous_body_dx, previous_body_dy, previous_delta_yaw, left_mps, right_mps | left_mps / right_mps are the commanded RPM divided by the gain 4524 to get m/s. |
| Target ×3 | target_body_dx, target_body_dy, target_delta_yaw | Body-frame next-step motion; the training objective. |

The rpm→m/s gain  left_mps = command_left_rpm / 4524, right_mps = command_right_rpm / 4524. The constant rpm_to_mps_gain = 4524 converts track RPM to a linear track speed. It maps the ≈±3,500 RPM range to ≈±0.78 m/s. This is a calibration constant in config.yaml; if the vehicle's gearing/wheel radius changes, this number must change too.

## 3.4  Quality filtering — how rows get rejected

The manifest builder (inspect_dataset.py → manifest.py) scans every samples.csv and drops any row that fails a check. A row can fail for several reasons at once. The filters:

| Rejection reason | Trigger |
| --- | --- |
| nonfinite_numeric | Any required numeric field is NaN/inf. |
| dt_too_short / dt_too_long | dt_sec outside [0.08, 0.12] s — i.e. the 10 Hz cadence hiccuped. |
| no_previous_delta | has_previous_delta is False (the first row of each run, mostly). Controlled by require_previous_delta. |
| rgb_sync_error | rgb_sync_error_ms > 40 ms — image too far from the step time. |
| missing_rgb / corrupt_rgb | The referenced PNG is absent or won't open/verify. |
| control_not_permitted / emergency_stop | Only when control_source = arbitrated: drops non-permitted and e-stop rows. |

Across the 18 experiments this removed 115 of 8,232 rows (~1.4%), leaving 8,117 valid rows. The dominant reason is no_previous_delta (the unavoidable first row of each run, plus a cluster of 51 in run 002), followed by a few dt_too_short and a handful of rgb_sync_error rows. No missing/corrupt images were found.

## 3.5  Splits and normalization

Splitting is done by whole experiment, never by row — an entire run is assigned to train, val, or test, so no run leaks across splits (a frame from a run can't appear in two sets). Assignment is deterministic: each experiment ID is hashed with the seed (42) and ranked, then sliced by the configured ratios. The result is saved to splits.json.

| Split | Experiments | Rows | Purpose |
| --- | --- | --- | --- |
| train | 12 | 5,378 | Fit the weights. |
| val | 4 | 1,818 | Early-stopping / pick best.pt. |
| test | 2 | 921 | Held-out final check (runs 101 & 006). |

Two config files, two split ratios  
      config.yaml uses ratios 0.70 / 0.15 / 0.15, while config_blueacer.yaml uses 0.70 / 0.20 / 0.10 and much smaller batch/worker counts (batch 4, 2 workers) for a lower-memory machine. The manifest currently on disk (12 / 4 / 2 experiments) matches the config_blueacer ratios. Everything else in the two files is identical. Use the blueacer file on the constrained “blueacer” machine; use config.yaml otherwise — but be aware they produce different splits, so regenerate the manifest if you switch.

Normalization statistics (mean and standard deviation) are computed on the training rows only and stored in normalization.json. Both the 5 inputs and the 3 targets are standardized to zero mean / unit variance before they touch the network; predictions are de-normalized back to physical units for reporting. Computing stats on train-only avoids leaking test statistics into training.

# 4. The model architecture

The model (vit_motion/model.py, class ViTMotionModel) is a “one image token + K numeric tokens” Transformer. Conceptually it has three stages: (1) turn the newest image into one visual token, (2) turn each of the K numeric steps into a token, (3) let a small Transformer mix all K+1 tokens over time and read out the answer from the most recent step.

## 4.1  Token construction

- Visual token (1): the latest RGB frame goes through a vit_tiny_patch16_224 ViT (from the timm library, ImageNet-pretrained), average-pooled to a single 192-d feature, then projected to a 64-d token via Linear → LayerNorm → GELU.

- Numeric tokens (K = 5): each step's 5 numbers are projected by a separate Linear → LayerNorm → GELU into a 64-d token. That yields 5 numeric tokens.

- Sequence assembled: [ visual token, numeric₁, numeric₂, …, numeric₅ ] = 6 tokens, each 64-d.

Two learned embeddings are added so the Transformer knows order and kind:

- Positional embedding (length K+1 = 6): which slot a token occupies.

- Token-type embedding (2 types): “visual” vs “numeric,” so the model can treat the image token differently from the state tokens.

## 4.2  Temporal Transformer and the head

The 6 tokens pass through a 2-layer Transformer encoder (temporal_d_model=64, nhead=4, dim_feedforward=128, dropout=0.10, GELU activation, pre-norm). Self-attention lets each token look at every other token — the visual context can inform the numeric steps and vice-versa, across all K time slots.

After encoding, the model takes only the last token (the most recent numeric step) and passes it through a small regression head (Linear 64→128 → LayerNorm → GELU → Dropout → Linear 128→3) to produce the three outputs [Δx, Δy, Δyaw] in normalized units.

Forward pass, with tensor shapes (batch B, K = 5):

image            [B, 3, 224, 224]
  → ViT encoder                       → [B, 192]
  → visual_projection (→LN→GELU)      → [B, 64]      (1 visual token)

      
numeric          [B, 5, 5]
  → numeric_projection (→LN→GELU)     → [B, 5, 64]   (5 numeric tokens)

      
tokens = concat(visual, numeric)      → [B, 6, 64]
tokens += positional_emb + type_emb
  → TransformerEncoder (2 layers)     → [B, 6, 64]
  take last token  encoded[:, -1, :]  → [B, 64]
  → motion_head                       → [B, 3]   = [Δx, Δy, Δyaw]

## 4.3  Sequence windows (how K steps are formed)

The dataset (dataset.py) slides a length-K window across each experiment. A window is only kept if:

- it lies entirely within one experiment (windows never cross run boundaries), and

- its sample_index values are contiguous (no gap left by a rejected row — the window won't jump over a hole).

Because the first prediction needs K steps of history, the first K−1 rows of every run are not usable as targets. The image, the target, and the output timestamp are always taken from the last row of the window, so the picture, prediction, and ground truth are never misaligned.

## 4.4  The deployment interface

The model exposes two methods that split image encoding from prediction:

- encode_image(image) → one visual token. Run the (expensive) ViT once.

- predict_from_feature(feature, numeric_sequence) → accepts [B, K, 5] and returns the delta. Reuses a cached visual token.

This separation exists so a controller running an MPC horizon can encode the image once and evaluate many candidate numeric roll-outs against the same visual feature. The README notes that a configurable “image refresh every M steps” is not yet part of the training semantics — v2 always uses the latest image — and should only be added once the model is validated.

# 5. Training setup

Training (train.py) is a standard supervised-regression loop with a few deliberate choices:

| Aspect | Choice & rationale |
| --- | --- |
| Loss | Weighted Smooth-L1 (Huber) on normalized targets. Robust to outliers vs plain MSE. Per-axis weights [1,1, yaw_weight]; yaw_weight = 1.0 currently (a knob to prioritize heading). |
| Optimizer | AdamW with two parameter groups: the pretrained ViT encoder trains at 0.1× the base LR (encoder_learning_rate_scale) so the pretrained features are fine-tuned gently, while the new heads train at the full 1e-4. |
| Regularization | weight_decay 1e-4; dropout 0.10 in the Transformer and head; light image augmentation (color jitter, occasional blur) on the training split only. |
| Mixed precision | AMP autocast + GradScaler on CUDA for speed/memory (no-op on CPU). |
| Schedule | Up to 100 epochs, batch size 32 (4 on blueacer), early stopping with patience 15 on validation loss. |
| Checkpoints | Saves last.pt every epoch and best.pt when val loss improves; history.json logs per-epoch metrics. The full config is embedded in each checkpoint. |

Validation metrics are reported both as the training loss and as physical mean-absolute-error per axis (mae_dx_m, mae_dy_m, mae_dyaw_rad), so you see error in meters and radians, not just normalized units.

Checkpoint compatibility  v2 checkpoints embed sequence_length. A v1 (single-step) best.pt is not loadable by v2 — the loader raises a clear error. Training and inference must use the same K. After any change to K, you must retrain; the manifest and normalization files, however, stay valid because the raw column definitions didn't change.

# 6. Evaluation & visualization tools

Four helper scripts turn a trained checkpoint into human-readable results:

| Script | What it produces |
| --- | --- |
| evaluate_experiment.py | Runs inference over one full experiment; writes predictions.csv, metrics.json (MAE, RMSE, R², and a MATLAB-style Fit %), and two figures: a 3-panel prediction-vs-ground-truth time series and an accumulated-displacement path (diagnostic). |
| make_validation_video.py | Builds an MP4: the RGB frame on the left, and on the right a panel showing the latest numeric inputs plus predicted vs true outputs with per-step error. Handles Unicode/UNC image paths. |
| plot_training_loss.py | Plots train vs validation loss from history.json and marks the best epoch. |
| smoke_test.py | Fast end-to-end sanity check: loads the train set, builds the model, runs one forward pass, asserts all shapes. Passing this means the pipeline is wired correctly. |

The validation output preserves each row's real time from the experiment start, and the first output row corresponds to the K-th valid, contiguous sample — so plots line up with the original run's clock.

# 7. Design decisions that remove ambiguity

Collected here are the non-obvious rules that, if missed, cause silent errors:

- RGB only. Depth images exist and are synchronized but are deliberately never read by the model.

- One image per window. Only the newest frame is decoded; the K history is purely numeric. This keeps the ViT cost at once-per-sample.

- Windows never cross runs or gaps. A rejected middle row splits the sequence; the window will not span the hole.

- First K−1 rows are never targets. They lack full history; expect each run to contribute (valid_rows − 4) usable windows.

- Split by experiment. No row-level leakage; a run is entirely in one split.

- Normalize on train only. Val/test use the training mean/std; targets are de-normalized for reporting.

- Use the command control source. RS485 feedback is negated and lagged; arbitrated is an option but filters e-stop rows.

- Keep K consistent everywhere. config, training, checkpoint, and inference must agree on K = 5.

# 8. Current status & recommended next steps

| Component | Status | Evidence |
| --- | --- | --- |
| Raw data (18 runs) | Present / verified | quality_report.json: 8,232 rows, 8,117 valid, 0 missing images. |
| Manifest / normalization / splits | Generated | artifacts/manifest/*.json + manifest.csv exist. |
| Model + training + tools | Complete | All source files present; smoke test defined. |
| Trained checkpoint (best.pt) | MISSING | artifacts/runs/vit_motion_v2/ is empty — no training has run. |
| Accuracy numbers | None yet | No metrics.json / history.json on disk. |

Recommended sequence:

- Confirm environment and data reachability: python inspect_dataset.py --config config_blueacer.yaml (regenerates the manifest against the live data root).

- python smoke_test.py --manifest-dir artifacts\manifest --sequence-length 5 — verify shapes.

- python train.py --config config_blueacer.yaml — produce best.pt and history.json.

- python evaluate_experiment.py … --experiment 20260730_afternoon_wet_006 then make_validation_video.py on a held-out (test) run — 006 and 101 are the test set.

- Read the per-axis MAE (meters / radians) and the Fit %; if heading is weak, raise yaw_loss_weight.

Open item flagged in the code  Deployment-side niceties — ViT feature caching, an “image age” signal, and a configurable image-refresh interval M — are intentionally deferred until the base model is proven. Don't add them before you have a working best.pt and validation plots.

# Appendix A — File inventory

| Path | Role |
| --- | --- |
| vit_motion/model.py | ViTMotionModel: ViT + Temporal Transformer + head. |
| vit_motion/dataset.py | MotionDataset + sliding K-window logic + normalization apply. |
| vit_motion/manifest.py | Column definitions, per-row quality filters, split hashing, normalization stats. |
| vit_motion/validation.py | Per-experiment inference, metrics (MAE/RMSE/R²/Fit%), checkpoint loading. |
| vit_motion/config.py | YAML loader + path resolution relative to the config file. |
| train.py | Training loop, loss, optimizer groups, AMP, early stopping, checkpoints. |
| inspect_dataset.py | Builds manifest.csv, normalization.json, splits.json, quality_report.* |
| evaluate_experiment.py / make_validation_video.py / plot_training_loss.py / smoke_test.py | Evaluation, video, loss plot, sanity check. |
| config.yaml / config_blueacer.yaml | Full-machine vs low-memory configs (differ in split ratios, batch, workers). |
| artifacts/manifest/* | Generated manifest, normalization, splits, quality reports. |

# Appendix B — Key numbers

| Quantity | Value |
| --- | --- |
| Experiments / total rows / valid rows | 18 / 8,232 / 8,117 (~1.4% filtered) |
| Split (experiments) | train 12 · val 4 · test 2 |
| Split (rows) | train 5,378 · val 1,818 · test 921 |
| Sequence length K | 5 |
| Model inputs / outputs | 1 image + 5 numeric per step / 3 deltas |
| Visual backbone | vit_tiny_patch16_224 (pretrained, 192-d feature) |
| Temporal Transformer | d_model 64 · heads 4 · layers 2 · ff 128 · dropout 0.10 |
| Image size / normalization | 224×224 · ImageNet mean/std |
| rpm→m/s gain | 4524 |
| dt acceptance window | 0.08 – 0.12 s (target 0.1 s / 10 Hz) |
| RGB sync tolerance | 40 ms |
| Loss / optimizer | weighted Smooth-L1 / AdamW (encoder LR ×0.1) |
| LR · weight decay · epochs · patience | 1e-4 · 1e-4 · 100 · 15 |

Report generated from the source code and artifacts in Tasks/04. All figures (row counts, ranges, config values) were read directly from the files listed in Appendix A.